Start |Author Index | View Uploaded Presentations | Meeting Information

# Joint 52nd Northeastern Annual Section / 51st North-Central Annual Section Meeting - 2017

Paper No. 25-13

Presentation Time: 1:30 PM-5:30 PM

# PHOSPHATE SORPTION ON KYANITE: PROGRESS TOWARDS A GEOENVIRONMENTAL TECHNOLOGY TO COMBAT NON-POINT SOURCE POLLUTION

VANGALA, Sunitha, Department of Geology & Environmental Earth Science, Miami University, Oxford, OH 45056, ABBOTT, Grant, Geology & Environmental Earth Science, Miami University-Hamilton, 1601 University Blvd., Hamilton, OH 45011, BRETZ, Richard L., Chemistry & Biochemistry, Miami University, 367 Hughes Hall, 651 E. High Street, Oxford, OH 45056 and KREKELER, Mark P.S., Department of Geology & Environmental Earth Science, Miami University-Hamilton, Hamilton, OH 45011, vangals@miamioh.edu

Phosphate non-point source pollution is a global environmental problem that largely impacts freshwater ecosystems causing extensive eutrophication and has negative impacts on marine ecosystems nearshore. Phosphate dominantly is derived from agricultural activities as well as human and animal sewage which is introduced to ecosystems primarily via overland flow or in the case of sewage direct or indirect discharge. This large scale phosphate pollution could be remediated if inexpensive technologies could remove or retard phosphate in drainage systems. The present study is centered on utilizing the mineral kyanite as a reactive sorption media for sequestering phosphate in freshwater systems. Kyanite is an aluminosilicate mineral that is used extensively in the ceramics industry but has not been investigated for environmental technologies. For this project three grain sizes of kyanite from Kyanite Mining Corporation in Virginia were investigated to determine sorption properties and utility as an environmental technology to control phosphate pollution. Materials were characterized by scanning electron micrscopy, transmission electron microscopy, and powder X-ray diffraction. Batch experiments were conducted using three different sizes of kyanite: K48 (48 mesh), K200 (200 mesh), and micronized kyanite (~5-7 micrometers). The concentrations for batch experiments were 2.5 ppm, 5.00 ppm, 10 ppm, 15 ppm, 20 ppm, 25 ppm, 30 ppm, 35 ppm, 40 ppm, 50 ppm, 60 ppm, 70 ppm and 80 ppm at pH values of 5.4, 6.2, and 7.0 which are appropriate for freshwater systems globally. Thirteen phosphate solutions were made from a stock solution using KH2PO4. Each solution was combined with one gram of each grain size and left to mix on a shake table for 24 hours. Then triplicate samples from each bottle were run through an ion chromatograph to determine the phosphate concentration. All isotherm plots show linear adsorption and preliminary analysis shows potential for utilizing kyanite as a reactive media. The 200 mesh kyanite material performed as well and sometimes better than the micronized kyanite. The results suggest kyanite is a viable geomaterial to remove phosphate from freshwater.

Session No. 25--Booth# 27

T40. Applied Geology, Environmental, Engineering, Hydrogeology, and Applied Geophysics (Posters)

Sunday, 19 March 2017: 1:30 PM-5:30 PM

Grand Ballroom, 17th Floor (Omni William Penn Hotel)

Geological Society of America Abstracts with Programs. Vol. 49, No. 2 
doi: 10.1130/abs/2017NE-290218

© Copyright 2017 The Geological Society of America (GSA), all rights reserved. Permission is hereby granted to the author(s) of this abstract to reproduce and distribute it freely, for noncommercial purposes. Permission is hereby granted to any individual scientist to download a single copy of this electronic file and reproduce up to 20 paper copies for noncommercial purposes advancing science and education, including classroom use, providing all reproductions include the complete content shown here, including the author information. All other forms of reproduction and/or transmittal are prohibited without written permission from GSA Copyright Permissions.

Back to: T40. Applied Geology, Environmental, Engineering, Hydrogeology, and Applied Geophysics (Posters)

<< Previous Abstract | Next Abstract >>