Start |Author Index | View Uploaded Presentations | Meeting Information

# Joint 52nd Northeastern Annual Section / 51st North-Central Annual Section Meeting - 2017

Paper No. 48-9

Presentation Time: 1:30 PM-5:30 PM

# A PRELIMINARY FIELD EMISSION SCANNING ELECTRON MICROSCOPY (FESEM) AND TRANSMISSION ELECTRON MICROSCOPY (TEM) FORAY INTO STREET SEDIMENTS OF GARY, INDIANA: MAJOR ENVIRONMENTAL HEALTH CONCERNS ARE EVIDENT

VANGALA, Sunitha1, DIETRICH, Matthew2, BURKE, Michelle3, WOLFE, Amy3, ARGYILAN, Erin P.4 and KREKELER, Mark P.S.5, (1)Department of Geology & Environmental Earth Science, Miami University, Oxford, OH 45056, (2)Geology & Environmental Earth Science, Miami University-Hamilton, 1601 University Blvd., Hamilton, OH 45011, (3)Department of Geology & Environmental Earth Science, Miami University, 250 S. Patterson Avenue, 114 Shideler Hall, Oxford, OH 45056, (4)Dept. of Geosciences, Indiana University Northwest, 3400 W. Broadway, Gary, IN 46408, (5)Department of Geology & Environmental Earth Science, Miami University - Hamilton, 1601 University Blvd., Hamilton, OH 45011, vangals@miamioh.edu

A current and ongoing urban geochemistry investigation of bulk chemical composition of street sediment from Gary, Indiana indicates that extensive metal pollution occurs (Dietrich et al., 2017, this meeting). Street sediment can be easily mobilized by wind and thus can expose residents to fine particulate matter through inhalation if particles are <PM10 and can enter the deep lung if particles are <PM2.5. This fine fraction can also be easily introduced into the local drinking water supply of Lake Michigan through runoff. The nature and diversity of phases of metal pollution in the fine fraction of street sediment in Gary are unknown as are the interactions of pollutant metals with natural minerals and geomaterials. Field emission scanning electron microscopy (FESEM) investigations and transmission electron microscopy (TEM) investigations were conducted to assess the nature of metal pollutants. Both SEM and TEM identify abundant technogenic particles with appreciable metal content. The diameters of these particles vary in size from several hundred micrometers to a few nanometers. Spherules, subhedral to euhedral oxides and metal fragments are very common, especially in the magnetic fraction of selected samples. Mn, Zn, Ni, Pb, Ni, Co and Cr are common constituents in technogenic particles at concentrations of several weight percent. Spherules are very common and vary in average diameter from approximately 100 micrometers to 200 nanometers. Mn is nearly ubiquitous in spherules regardless of size and suggests a common source. Considering bulk chemical geospatial data trends, the source of Mn-enriched spherules is very likely the local steel manufacturing facility. This is the first electron microscopy investigation of street sediment of Gary and initial results indicate a high degree of complexity. Electron microscopy shows promise explaining variations of metal pollutants geospatially as well as covariation internal to the bulk geochemical data set.

Session No. 48--Booth# 32 T24. Urban Biogeochemistry and Geochemistry

Monday, 20 March 2017: 1:30 PM-5:30 PM

Grand Ballroom, 17th Floor (Omni William Penn Hotel)

Geological Society of America Abstracts with Programs. Vol. 49, No. 2 
doi: 10.1130/abs/2017NE-290614

© Copyright 2017 The Geological Society of America (GSA), all rights reserved. Permission is hereby granted to the author(s) of this abstract to reproduce and distribute it freely, for noncommercial purposes. Permission is hereby granted to any individual scientist to download a single copy of this electronic file and reproduce up to 20 paper copies for noncommercial purposes advancing science and education, including classroom use, providing all reproductions include the complete content shown here, including the author information. All other forms of reproduction and/or transmittal are prohibited without written permission from GSA Copyright Permissions.

Back to: T24. Urban Biogeochemistry and Geochemistry (Posters)

<< Previous Abstract | Next Abstract